Systems II – Reliability Engineering

Definitions

- Operability – the ability of a plant to perform satisfactorily under conditions different from the nominal design conditions

  - Feasibility of steady-state operation fro a range of different feed conditions and plant parameter variations (flexibility)

  - Fast and smooth changeover and recovery from process disturbances (controllability, resiliency)

  - Safe and reliable operation despite equipment failures

  - Easy star-up and shut-down

- Reliability – probability that an item performs satisfactorily its specified function over a given time interval when operating under stated conditions

- Availability – probability that an item is performing satisfactorily at any point in time when operating under stated conditions and taking into account reliability and repair

- Maintenance – includes all actions to return a system from failed state to an operating or available state (corrective), or to retain a system in an operational (available) state by preventing failures (preventive)

  - Maintenance costs may form 20-30% of the operating budget of a manufacturing company

  - Maintenance optimisation aims in achieving a proper balance between maintenance costs and the benefit of increased process availability

- System effectiveness – the ratio between actual output (availability × capacity) and demand

Basic Concepts

## Probability

- Defines quantitatively the likelihood of an event

Psuccess=number of successful eventsnumber of possible outcomes

Psuccess+Pfailure=1

## Probability Distribution Functions (see handout):

- Discrete

  - Binomial

  - Poisson

- Continuous

  - Exponential

  - Weibull

  - Normal

## Conditional Probability

BS

BF

A

F = Failure

S = Success

BF, BS = mutually exclusive

### Bayes’ Theorem:

PA=PABF×PBF+PABS×PBS

## Time

- t = 0

  - The component is known to be operating

  - Probability of failure at t = 0 is zero

- t  ∞

  - It is a certainty that a component will fail

  - Probability of failure is unity

## Reliability

- The probability that a component does not fall under specified operating conditions during the time interval (0, t) given that it was as good as new at time t=0

Rt=Total number surviving to age tTotal population (sample)=NStN

t

R(t)

1.0

0

Reliability Distribution

## Unreliability

- The probability that a component fails for the first time during the interval (0, t) given that it was as good as new at time t=0

Ft=Total number of failures before age tTotal population (sample)=NftN

Note:	Rt+Ft=1

F(t)

t

1.0

0

Unreliability Distribution

## Failure Density

### Continuous case:

ft=dFtdt

### Discrete case:

ft=Nft+Δt−NftN⋅Δ

Where the numerator is the number of failures in the time interval 

f(t)

t

0

F(t)

R(t)

Failure Density

## Failure Rate

- Probability of failure per unit time t given that the component was “repaired” at time zero and has survived to time t

rt⋅Δ⋅No. of components survived up to t=No. of failures during time interval Δ

		No. of components survived up to t=Nst=Rt⋅N

		No. of failures during time interval Δ=Nft+Δt−Nft=ft⋅N⋅Δ

		rt⋅Δ⋅Rt⋅N=ft⋅N⋅Δ

		rt=ftRt=ft1−Ft=dFtdt⋅11−Ft			(1)

## Relationships

By integrating (1) and rearranging, we obtain three important relationships:

### Unreliability:

Ft=1−exp−rtdt

### Reliability:

Rt=1−Ft=exp−rtdt

### Failure Density:

ft=dFtdt=rt⋅exp−rtdt

## Mean Time to Failure (MTTF)

- Denotes the expected value of the failure density function f(t). Defined by the first moment of the failure density

MTTF=t⋅ftdt

OR		MTTF=Rtdt

t

f(t)



0

1/

0.368

Constant Failure Rate Model

- Failure rate = r(t) = constant = 

Ft=1−exp−λt

		Rt=exp−λt

		ft=λexp−λt

		MTTF=1λ

		fMTTF=0.63

## Weibull Distribution

- Failure rate:

rt=βαtαβ−1			(,  0; t  0)

Where	 is the shape parameter

 is the scale parameter

- Note: The failure rate of the Weibull distribution is increasing in t for values of >1, decreasing in t for values of <1 and constant when =1. Thus, the exponential distribution (constant failure rate model) is a special Weibull distribution with =1

Ft=1−exp−tαβ					t>0

Rt=exp−tαβ						t>0

ft=βαtαβ−1⋅exp−tαβ			t>0

- Unlike exponential model, Weibull distribution is applicable to a variety of failure scenarios. Can be used to monitor situations where a component is either deteriorating or improving with usage

## Three-parameter Weibull Distribution

- Failure Density:

ft=βαt−ϕαβ−1⋅exp−t−ϕαβ	t>

Since:	Ft=ftdt

		Ft=1−exp−t−ϕαβ

And:	Rt=1−Ft

		Rt=exp−t−ϕαβ

Now:	rt=ftRt

		rt=βαt−ϕαβ−1

## Relationship between different Reliability functions

|  | Failure Density | Failure Rate | Unreliability | Reliability |
| --- | --- | --- | --- | --- |
| f(t) = | ft | λtexp−0trξdξ | ddtFt | −ddtRt |
| r(t) = | ft1−0tfξdξ |  | 11−FtddtFt | −ddtlnRt |
| F(t) = | 0tfξdξ | 1−exp−0trξdξ | Ft | 1−Rt |
| R(t) = | 1−0tfξdξ |  | 1−Ft | Rt |

## Availability, Unavailability and Repair

### Availability:

At=No. of normal components at tTotal population (sample)

### Unavailability:

Qt=No. of failed components at tTotal population (sample)

- Repair – contributes to availability but NOT to reliability:

At≥Rt

For non-repairable components

At=Rt

- Repair Distribution – Probability that a component is repaired before time t given that it failed at time t=0:

Gt=No. of components repaired before tNumber of componentes whose  repair started at t=0

- Repair Density – Probability that component repair is completed during time interval (t, t) given that it failed at time t=0:

gt=dGtdt

- Repair Rate – Probability that component is repaired per unit time at time t given that it failed at time t=0 and was in a failed state up to t

- Mean Time to Repair – Expected time to repair:

MTTR=0∞t⋅gtdt

- Mean Time between Failures – For repairable components:

MTBF=MTTR+MTTF

## Constant Repair Rate Model

- Repair rate = m(t) = constant = 

Gt=1−exp−μ⋅t

		gt=μ⋅exp−μ⋅t

		MTTR=1μ

Combinatorial Aspects of Systems Reliability

SEE NOTES