EXTENTS OF REACTION

Consider the following reaction:

aA+bB+cC→xX+yY+zZ

- Batch Reactor

xA=NA0−NANA0,xB=NB0−NBNB0,xC=NC0−NCNC0

Define Extent of Reaction, :

ξ≡Ni−Ni0νi		[kmol]		dξ=dNiνi  dNi=νidξ

Where	νA=−a,   νB=−b,   νC=−c

And:	νX=x,   νY=y,   νZ=z

Hence:	ξ=NA−NA0−a=NA0−NAa, etc.

NOTE:  has the same value for every component

## Mass balance:

rA=−1VRdNAdt	with	xA=NA0−NANA0

Substituting dNi=νidξ:

rA=−νAVRdξdt		(*)

The rate of reaction rA can also be expressed in terms of :

rA=kCA=kNAVR=kVRNA0+νAξ		(**)

Equating (*) and (**):

kVRNA0+νAξ=−νAVRdξdt

	dξNA0+νAξ=−kνAdt

	1νAlnNA0+νAξ0ξ=−kνAt

	1νAlnNA0+νAξNA0=−kνAt

	NA0+νAξNA0=exp−kt

	ξ=NA0νAe−kt−1	or	ξ=−NA0νA1−e−kt

- Tubular Reactor using PFA

Derivation is nearly identical:

## Mass balance:

rA=−dnAdVR

Substituting dni=νidξ:

rA=−νAdξdVR	(*)

The rate of reaction rA can also be expressed in terms of :

rA=kCA=knAvT=kvTnA0+νAξ		(**)

Equating (*) and (**):

kvTnA0+νAξ=−νAdξdVR

	dξnA0+νAξ=−kνAvTdVR

	1νAlnnA0+νAξ0ξ=−kνAvTVR

	1νAlnnA0+νAξnA0=−kνAvTVR

	nA0+νAξnA0=exp−kτ		where	τ=VRvT

	ξ=nA0νAe−kτ−1	or	ξ=−nA0νA1−e−kτ

- CSTR

As before:

## Mass balance:

rA=nA0−nAVR

Substituting dNi=νidξ:

rA=−νAξVR		(*)

The rate of reaction rA can also be expressed in terms of :

rA=kCA=knAvT=kvTnA0+νAξ		(**)

Equating (*) and (**):

kvTnA0+νAξ=−νAξVR

	νAξ=−νAkτξ−nA0		where	τ=VRvT

	νAξ−νAkτξ=−nA0

	ξ=−nA0νA−νA/kτ

	ξ=−nA0kτνAkτ+νA

	ξ=−nA0νAkτ1+kτ

- Multiple reactions

The equation takes the form:

ni−ni0=j=1Jνijξj

Where:	j = extent of reaction j

J = total number of reactions

For component i we can write:

dni=dj=1Jνijξj=j=1Jνijdξj

For N components in the reaction mixture, conservation of mass requires:

j=1NMij=1Jνijξj=0

Pick a key reactant k for each reaction:

Batch Reactor				dξjdt=−VRrkjνkj

Tubular Reactor with PFA		dξjdVR=−rkjνkj

CSTR					ξj=−VRrkjνkj

## Example:

Set of reactions carried out in a PFR:

A→B+C		(1)	rA1=k1CA

2A→D		(2)	rA2=k2CA2

C↔D+E		(3)	rC=k3CC−k−3CDCE

Use the general equation above for a PFR to write mass balances:

dξ1dVR=−rA1−1=k1CA=k1nAvT

dξ2dVR=−rA2−2=12k2CA2=12k2nA2vT2

dξ1dVR=−rC3−1=k3CC−k−3CDCE=k3nCvT−k−3nDnEvT2

Now use definition of extent of reaction:

ni−ni0=j=1Jνijξj

To obtain the following equations:

nA=nA0+j=13νAjξj=nA0−ξ1−2ξ2

nC=nC0+ξ1−ξ3

nD=nD0+ξ2+ξ3

nE=nE0+ξ3

Substitute into the three differential equations and solved for 1, 2 and 3. If vT is not constant, must develop an expression for vT in terms of j.

NON-ISOTHERMAL REACTORS

Temperature is an important parameter even in the simplest reaction rate expressions:

rA=kCA=kTCA	where	kT=k0exp−Ea/RT

The rate constant increases rapidly with temperature. In the design of non-isothermal reactors:

- The material balance equations derived for isothermal reactors will remain largely unchanged.

- The dependence of reaction rates on temperature necessitates the simultaneous solution of the material and energy balance equations. This usually requires the solution of simultaneous non-linear differential equations.

- Energy Balance Equation

Define:	(KE)m = kinetic energy per unit mass

(PE)m = potential energy per unit mass

UTm = total energy per unit mass

W = shaft work done by the system on the surroundings

w = mass flowrate

Total internal energy per unit mass:

UTm=Um+KEm+PEm

Total enthalpy per unit mass:

HTm=UTm+PV=Um+PV+KEm+PEm

- CSTR

At steady state:

wHTm0−wHTm+W+Q=0			(1)

### Assumptions I:

- Changes in kinetic energy (due to changes in the velocity of the reactor) relatively small, even though value of KE may be large.

- Similarly, changes in potential energy (related to gravity effects) may usually be neglected.

- Shaft work: strictly we must take account of the mechanical energy that the stirrer imparts to the reaction mixture. In practice however, the amount of energy involved is usually negligible compared to reaction related heat effects (i.e. heat of reaction).

Hence:		ΔKE≈0	ΔPE≈0	W≈0

So the energy balance equation for a steady-state system becomes:

wHm0−wHm+Q=0				(2)

At steady-state, the mass flowrate w is constant. Generally more practical to work with molar flowrates:

wHm=nTH

Which means that (2) is now:

nTH0−nTH+Q=0				(3)

Assume that heats of mixing and pressure effects on the enthalpy are negligible (i.e. dealing with perfect gas or ideal solution). Hence:

nTH=niHf,i

For any component ‘i’ the enthalpy Hf,i can be found from:

Hf,iT=Hf,i0+CpdT

Substituting into the energy balance equation:

ni0Hf,iT0−niHf,iT+Q=0		(4)

Note: non-isothermal behaviour refers to differences between the feed temperature (T0), the reaction temperature (T) and heat input/removal during the reaction. The reactor is isothermal within the reaction zone.

Recall the mass balance for a CSTR:

ni0−ni=VRri			ni=ni0−VRri

Substituting it into the energy balance:

	ni0Hf,iT0−ni0−VRriHf,iT+Q=0

	ni0Hf,iT0−ni0Hf,iT−VRriHf,iT+Q=0

	ni0Hf,iT0−Hf,iT−VRriHf,iT+Q=0

We can simplify further by using the expression:

ni0Hf,iT0−Hf,iT=−ni0Cp,idT

Rewriting the energy equation:

−ni0Cp,idT+VRHf,iTri+Q=0		(5)

In the case of a single reaction taking place, A→B+C:

Hf,iri	=Hf,A+Hf,BrB+Hf,CrC	rA=−rB=−rC

=rAHf,A−Hf,B−Hf,C

=−rAΔHr

Then the energy balance in (5) takes the form:

−ni0Cp,idT+VRΔHrrA+Q=0		(6)

### Assumptions II:

- Using Cp,i=1ΔTCp,idT

- And: nT0Cp=ni0Cp,i

Reduces equation (6) to:

nT0CpT0−T+VRΔHrrA+Q=0			(7)

Note: heat ‘in’ is defined as (+); heat ‘out’ is defined as (-).

## Example:

Consider the liquid phase reaction A  products in a CSTR.

- What is the adiabatic operating temperature?

- What is the operating temperature if heat is withdrawn using a coolant at 300 K?

Data:

A = 2 m2

U = 1200 J/s.m2.K

CA0 = 10 mol/litre

vT = 0.5litre/s

xA = 0.9

T0 = 300 K

Hr = -100 kJ/mol A

Cp = 4.2 kJ/litre.K

a)	Qg=nA0xA−ΔHr=vTCA0xA−ΔHr

No heat removed by cooling, so only heat removal is by flow:

Qr=vTCpT−T0

At steady-state, Qg = Qr:

vTCA0xA−ΔHr=vTCpT−T0

Solving for T (=T-T0):

ΔT=CA0xA−ΔHrCp=10×0.9×100,0004,200=214.3 K

Hence the adiabatic operating temperature is: 300 + 214.3 = 514.3 K

Clearly too high to keep things liquid, MUST remove heat from the reactor.

b)	Qg=nA0xA−ΔHr=vTCA0xA−ΔHr

Qr=vTCpT−T0+UAT−Tc		Tc=300 K

Equating and solving for ΔT=vTCA0xA−ΔHrvTCp+UA=0.5×10×0.9×100,0000.5×4,200+1,200×2=100 K

So the operating temperature goes down to: 300 + 100 = 400 K

## Multiplicity of steady-states

Comparison of the magnitudes of terms within the equation:

nT0CpT0−T−VRΔHrrA+Q=0

Consider the irreversible first order reaction being carried out in a CSTR:

A→R+S

### Heat Generation:

Qg=VRΔHrrA=VrΔHr−kCA=−kVRvTnAΔHr

	Qg=−kτnAΔHr	(where k=k0exp−Ea/RT and τ=VR/vT)

Recall that for CSTR:	nA=nA01+kτ

Hence can rewrite Qg as:

Qg=−kτΔHrnA01+kτ=−nA0ΔHr1+eEaRT/τk0

For and exothermic reaction, where ΔHr<0

T

Qg

Qg

### Heat removal:

Qr=Q(due to heat exchange)+Q(due to flow)

For adiabatic operation, Q (due to heat exchange) = 0 and:

Qr=nT0CpT−T0

For non-adiabatic operation, using a simple heat transfer term:

Qr=nT0CpT−T0+UAT−Tc

	Qr=nT0Cp+UAT−nT0CpT0+UATc

So for this simple model Qr is linear in T.

Plot and compare the magnitudes of the heat generation and hear removal terms of the energy balance equation:

T

Qg

Qr

or

Q1,r

Q1,r

Q1,r

Qg

Clearly, at steady-state, Qg = Qr – the intersections are the only possible operating points.

For a reversible reaction, the shape of the heat generation cure would take the form:

T

Qg

Qr

or

Qg

For two consecutive exothermic reactions and two consecutive reactions, first exothermic and second endothermic:

T

Qg

Qr

or

Qg

T

Qg

Qr

or

Qg

## Adiabatic Operating Line:

Assume a first order irreversible reaction A  B and write the material balance equation:

VR=nA0−nArA=nA0−nAkCA=nA0−nAknA/vT

	τ=nA0−nAknA			(nA=nA01−xA

T

xA

	τ=nA0xAknA01−xA

	xA=kτ1+kτ

	xA=11+eEART/τk0	(1)

The corresponding energy balance equation is:

nT0CpT0−T−VRΔHrrA+Q=0

For adiabatic operation:

nT0CpT0−T−VRΔHrrA=0

Since:	VRrA=nA0−nA	and	nA0−nA=nA0xA

	nT0CpT0−T−nA0xAΔHr=0

Solving for conversion, xA:

	xA=−nT0CpnA0ΔHrT−T0

Recalling yA0=nA0/nT0

	xA=−CpyA0ΔHrT−T0	(2)

We can now plot both mass balance and energy balance equations together, where the intersections represent the simultaneous solutions of the mass and energy balances.

When we plot the energy:

Slope > 0 for exothermic reactions (Hr < 0 )

Slope < 0 for endothermic reactions (Hr > 0 )

NOTE: The slope of the material balance curve remains unchanged for Hr>0 or Hr<0

T

xA

Hr<0

Hr>0

T0

This is the treatment for a simple 1st order irreversible reaction. Considerably more complicated for a simple reversible reaction A  B.

The basic design problem consists in finding the most desirable intersection of the mass and energy balances. In this simple case the parameters that can be readily modified are:

yA0nA0/nT0	and	τVR/vT

- TUBULAR REACTORS

At steady-state, the energy balance over a volume element in terms of molar flow rates would be derived as follows:

nTHT|z−nTHT|z=Δz+W+qdz=0		(1)

Where z is the distance from the reactor inlet.

### Assumptions I:

- Plug flow

- Neglect changes in KE and PE

- Neglect shaft work, W

Rearranging (1):

−nTHT|z−nTHT|z+Δz+qΔz=0			(2)

Taking the limit as Δz→dz and q→qz, i.e. a point value:

−ddznTHT+qz=0				(3)

And by multiplying by A, the cross sectional area:

−ddVRnTHT+Qz=0				(4)

### Assumptions II:

- Neglect the enthalpy change upon mixing (perfect solution)

- Neglect effect of pressure changes in enthalpy (perfect gas)

Equation (4) then takes the form:

−ddVRniHf,i+Qz=0		(5)	where	Hf,i=ΔHf,i0+dHi

Now rewrite the LHS term as:

ddVRnTHf,i=nidHf,idVR+Hf,idnidVR

Recalling that dHf,i=Cp,idT and that in a tubular reactor ri=−dni/dVR:

ddVRnTHf,i=niCp,idTdVR−Hf,iri

	ddVRnTHf,i=dTdVRniCp,i−Hf,iri

For a single reaction taking place, A  B:

Hf,iri=Hf,ArA+Hf,BrB		but	rA=−rB

	Hf,iri=Hf,ArA−Hf,BrA=rAHf,A−Hf,B=−ΔHrrA

Simplifying energy balance equation (5) to:

−dTdVRniCp,i+rAΔHr+Qz=0		(6)

Hence the general energy balance can be cast in the form of an ordinary differential equation, to be solved simultaneously with the material balance equation(s). Both temperature and conversion change as a function of position within the reactor:

dTdVR=Q−rAΔHrniCp,i					(7)

Where:	Q = 0 corresponds to adiabatic operation

Q = Q0 constant flux (electrical heating or furnace)

Qj = -AU(Tj-Tc) where Tj is local reactor temperature and Tc the coolant temp.

Assume that the heat capacity of the reaction mixture may be considered approximately constant over the temperature and concentration changes:

niCp,i≃nT0Cp

So equation (7) can be rewritten as:

dTdVR=Q−rAΔHrnT0Cp					(8)

## Mass balance:

In the case of a single reaction taking place, the energy balance equation will be solved coupled to a single mass balance equation:

rA=−dnAdVR=−ddVRnA)−nA0xA=nA0dxAdVR

	dxAdVR=rAnA0						(9)

The simultaneous solution of equations (8) and (9) usually requires resort to numerical solutions, even for the simplest rate expressions. In the simplest case, it is easier to work with Hr  f(T) and Cp  f(T) and an operating line on an xA versus T axis. Dividing the energy balance equation by the mass balance equation:

dTdxA=nA0nTQrACp+−ΔHrCp

	dTdxA=yA0QrACp+−ΔHrCp		since	yA0=nA0/nT

For adiabatic operation (assuming constant Hr and Cp over the temperature range), equation is linear with temperature and will give a curved line for Q  0:

	xA=−CpyA0ΔHrT−T0				(10)

T

xA

Q=0 (adiabatic)

Q<0 (heat removal)

NOTE: For CSTRs the intersection of the two lines for the material and energy balances allow pinpointing the operating point, whereas in a tubular reactor temperatures and concentrations change continuously.

Conversion and temperature for and exothermic reaction (i.e. H<0) in an adiabatic reactor:

Length along reactor

xA

T

Conversion and temperature for and exothermic reaction (i.e. H<0) with heat removal at the walls (non-adiabatic, non-isothermal reactor):

Length along reactor

xA

T

Conversion and temperature for and endothermic reaction (i.e. H>0):

Length along reactor

xA

T

## Example:

Irreversible gas-phase reaction is carried out in an adiabatic PFR:

A+B→2C			rA=kpApB

Calculate the reactor volume required for 80% conversion of that reactants

Data:

Inlet mole fractions: yA0 = 0.10; yI0 = 0.80 (inerts)

T0 = 300 K

nT0 = 8.9 x 10-4

PT = 1 bar

k = 2 x 106 exp(-3000/T) mol/s.m3.bar2	(T in K)

Hr = -60 kJ/mol A

Cp = 30 J/mol.K

Mass balance:

VR=−dnArA	where rA=kpApB=kpA2=k1−xA2pA02;	dnA=−nA0dxA

	VR=−dnAk1−xA2pA02=nA0dxAk0exp−3000/T1−xA2pA02

	VR=nA0pA02k0nA0dxA1−xA2exp−3000/T

With xA = 0.8

	VR=4.45×10−9dxA1−xA2exp−3000/T		(*)

T is no longer constant, function of conversion too. Need the energy balance equation to find a relationship between xA and T. Recall the general equation derived before:

dTdxA=nA0nTQrACp+−ΔHrCp		and setting Q=0 for adiabatic operation

	dTdxA=nA0nT−ΔHrCp=1×60×10310×30=200 K

	dT=200dxA

	T=T0+200xA

Substitute the result into (*) above:

	VR=4.45×10−9dxA1−xA2exp−3000/T0+200xA	(**)

Now need a numerical method for integrating the expression.

Use the Trapezoid Rule:

fxdx≈Δxf02+f1+f2+...+fn−1+fn2	where	Δx=x/n

In the integral (**), f(xA) takes the form:

fxA=11−xA2exp−3000/T0+200xA		T0=300 K;  ΔxA=0.8/8=0.1

Hence:	f02+f1+f2+...+fn−1+fn2=8.61×104

And:	VR=4.45×10−9fxAdxA≈ΔxAf02+f1+f2+...+fn−1+fn2

	VR=4.45×10−9×0.10×8.61×104=3.83×10−5 m3