REVERSIBLE REACTIONS IN NON-ISOTHERMAL REACTORS

Consider the reaction:

A↔k1k2B		where	rA=k1CA−k2CB

And defining the equilibrium constant as:

Keq=k1k2=CB,eqCA,eq

If we start with pure A:

Keq=CB,eqCA,eq=CA0xA,eqCA01−xeq=xA,eq1−xA,eq

	xA,eq=Keq1+Keq

- ARRHENIUS EQUATION FOR REVERSIBLE REACTIONS

The activation energy of a chemical reaction is defined by the slope of the straight line obtained by plotting ‘ln k’ against ‘1/T’ for a single reaction:

dlnkd1/T=−EaR

	dlnkdT=−EaRT2

Integrating this expression we obtain the Arrhenius equation:

k=k0exp−Ea/RT	R: gas constant=8.314 kJ/kmol.K=0.08314 bar.m3/kmol.K

- REVERSIBLE REACTIONS

dlnk1dT=−Ea1RT2	and	dlnk2dT=−Ea2RT2

Where:	Keq=k1k2⇒lnKeq=lnk1−lnk2⇒dlnKeq=dlnk1−dlnk2

Subtracting the two equations from one another:

dlnk1−dlnk2=dlnKeq=−Ea1−Ea2Rd1/T

	dlnKeqd1/T=−ΔHrR

	dlnKeqdT=ΔHrRT2

The form of the equation indicates that:

- For endothermic reactions (Hr>0)		as T		Keq 

- For exothermic reactions (Hr<0)		as T		Keq 

Both rate constant increase with temperature, but for Hr<0, k2 increases relative to k1 with increasing temperature.

In terms of our simple reaction	A↔k1k2B:

- For endothermic reactions (Hr>0)	Keq 		k1/k2 		as T

The rate of reverse reaction decreases RELATIVE to the forward reaction

T

xA,eq

At equilibrium:

rA=0

Thus for irreversible reactions or reversible endothermic reactions we would seek to operate at the highest temperature possible.

- For exothermic reactions (Hr<0)	Keq 		k1/k2 		as T

The rate of reverse reaction increases RELATIVE to the forward reaction

T

xA,eq

At equilibrium:

rA=0

Choice of operating conditions has to take into account two competing effects:

- Rates of reaction (both forwards and reverse) increase with rising temperature, so the kinetics is favoured by increasing temperature.

BUT

- Equilibrium limitations are more restrictive at higher temperatures: the maximum attainable conversion, xa,eq takes on smaller values as the temperature is increased as show in the graph above.

Let’s look at the xA vs. T diagram for a reversible exothermic reaction:

T

xA

xA,eq

Constant reaction rate lines

Constant xA line

Increasing rA

At constant reaction rates, the conversion initially rises rapidly with increasing temperature and then begins to fall – due to the effect of the reversible reaction.

The characteristics of the reversible exothermic reaction can be better understood by following a horizontal (constant conversion) line:

- At low temperatures, the reaction rate would be low, but would INITIALLY increase as you move to the right – towards higher temperatures.

- At the other extreme, the intersection of the constant xA line with xA,eq represents rA.

- Between these two extremes, the reaction rate would pass though a maximum.

- For a reversible exothermic reaction, there exists an “optimum” operating temperature (corresponding to the maximum reaction rate) for each fixed conversion.

- Remember: this discussion is independent of reaction vessels.

- THE LOCUS OF MAXIMUM REACTION RATES

For a simple reversible exothermic reaction it is easy to find the locus of maximum reaction rates for each conversion.

A↔k1k2B

Assuming CB0 = 0

rA=k1CA01−xA−k2CB=CA0k1−k1+k2xA

Where:	k1=k10exp−ΔE1/RT;	k2=k20exp−ΔE2/RT	and ΔE2>ΔE1

The maximum reaction rate as a function of temperature for a fixed conversion:

drAdT|xA=0=CA0dk1dT−CA0dk1dT+dk2dTxA

	0=dk1dT−dk1dT+dk2dTxA

	xA,opt=dk1dTdk1dT+dk2dT=11+−ΔE2/R−1/T2k20exp−ΔE2/RT−ΔE1/R−1/T2k10exp−ΔE1/RT

	xA,opt=11+ΔE2ΔE11KeqT

Having derived xA,opt (T) along the locus of maximum reaction rates, the corresponding ‘maximum reaction rates’ can be derived as a function of temperature:

rA=CA0k1−k1+k2xA

	rA,max=CA0k1−k1+k21+ΔE2ΔE11Keq=k1CA0k1−1+1Keq1+ΔE2ΔE11Keq

For a given conversion, the reactor volume would be a minimum if the temperature of operation were selected to be that corresponding to the maximum reaction rate.

NOTE: Both the constant rate lines and the locus of maximum reaction rates are independent of the type of reactor used.

Numerical example: see page 86 of the notes (Part IV)

Can now draw the operating line into the xA vs. T diagram:

T

xA

Operating line

xA,opt

xA,eq

T0 – reactor inlet

The interpretation of the operating line is different for PFRs and CSTRs.

- For PFRs: the intercept on the T-axis represents the reactor inlet conditions. Reactor operation may be followed by moving along the operating line towards increasing conversion. Every point along the line represents the set of conditions at a point within the reactor.

- For CSTRs: the intercept on the T-axis represents the reactor inlet conditions. Operates at a single point – somewhere on the operating line. By adjusting reaction parameters it is possible to operate at a point on the locus of maximum reaction rates.

- CONVERSIONS IN CSTRs

Consider the same reversible reaction:

A↔k1k2B

## Mass balance:

Vr=nA0−nArA=nA0xAk1nAvT−k2nBvT

Using	nA=nA01−xA and nB=nA0xA and assuming nB0=0:

	τ=nA0xAk1−k1+k2xAnA0

Solving for xA:

	xA=τk1−k1+k2τxA

	xA=τk11+k1+k2τ

	xA=Keq1/k2τ+Keq+1	[ k2]

Or	xA=Keq1/k1τ+1+1/Keq	[ k1]

As   	 equilibrium xA vs. T line:

Limτ→∞xA=Keq1+Keq

## Energy balance (endothermic reaction):

Adiabatic operating line given by:

xA=−CpyA0ΔHrT−T0

Where Hr>0 and the slope of the line is negative. Since a CSTR operates at a single point only, the operating line joins the inlet and operating points.

T

xA

2 (<1)

1

At equilibrium

rA=0

  

T0

Operating line

Numerical example: see page 91 of the notes (Part IV)

## Energy balance (exothermic reaction):

Adiabatic operating line given by:

xA=−CpyA0ΔHrT−T0

Where Hr>0 and the slope of the line is positive.

T

xA

xA,eq

T0

Operating line

1

2

1 > 2

Mass balance

equation

Numerical example: see page 93 of the notes (Part IV)

## Discussion:

T

xA

Operating line

rA,max

xA,eq (rA=0)

T0 – reactor inlet

For a CSTR, the intersection of the operating line (energy balance) and the locus of the optimum reaction rates indicates the optimum adiabatic operating point.

For a PFR the final point MUST be close to the highest achievable conversion (xA,eq) without getting too close to the equilibrium line.

Staging may be advantageous for both cases in order to minimise the total reactor volume. For CSTR the problem reduces to operating at several points along the locus of maximum reaction rates.

- INTER-STAGE COOLING AND COLD-SHOT

Try to take advantage of the fact that:

- At higher temperatures the rate is higher

- At lower temperatures (while the rate is low) the concentration driving force and attainable conversions are high.

Two stage-wise cooling methods:

- Inter-stage cooling

- Cold-shot cooling

Each stage is assumed adiabatic. Discussion applies to both CSTRs and PFRs.

## Inter-stage cooling:

Tf

T0

T1

T2

T3

Heat

Cool

Cool

Reactor I

Reactor II

Heat-exchangers are placed between the reactor stages, where the temperature is reduced without affecting the composition of the process stream (reactor stages are catalytic).

An inter-stage cooled reactor system allows working close to the locus of maximum rates. The larger the number of steps, the closer the trajectory of the system would approach the theoretic optimum.

T

xA

or

A

rA,max

xA,eq (rA=0)

T0

Tf

T1

T2

T4

T3

Reactor stages are assumed adiabatic. Energy balance :

nA0xAΔHr=nT0CpT−T0

Equation derived assuming total heat capacity of the mixture, Cp.nT0, remains constant even though the composition changes. Amount reacted is nA0.xA for the first stage and nA0(xA,i –xA,i-1) for each subsequent stage ‘i’.

xA=−nT0CpnA0ΔHrT−T0	or	xA,i−xA,i−1=−nT0CpnA0ΔHrT−T0

The slopes of the operating lines may be assumed to remain constant.

Size of the reactor is very important. Usually the first stage is designed to be smaller than later stages because the concentration driving force is larger and a large amount of heat is released in the initial stages of the reaction.

T

xA

or

A

rA,max

T0

Tf

T1

T2

T4

T3

Max. possible conversion

Need to solve the mass and energy balance equations in order to calculate reactor sizes (reactor volumes or amounts of catalyst).

## Cold-shot cooling:

T1<

T2

T3 >

T4

T5

(1-)

Tf

T0



Reactor I

Reactor II

Reactor III

1

2=-1

nT0(1-)

i=

Unlike inter-stage cooling, mixing cold fee with the process stream has the effect of not just cooling the mixture, but also adding fresh reactant and changing the process stream composition.

T

xA

or

A

rA,max

T0

Tf

Max. possible conversion

For the first stage, the operating line still has the form:

xA=−nT0CpnA0ΔHrT−T0

The operating line for each subsequent stage also has the same slope.

Good:

- No inter-stage heat-exchangers

Bad:

- For same number of stages  lower output  shorter average residence time

- Reactant bypass  lower overall conversion

- Limit to maximum conversion, for same feed temperature, is lower

NOTE: graphs do not contain information on the size of catalyst beds required. Information to come from simultaneous solution of mass and energy balance equations.

CSTR: possible to operate ON the locus of maximum reaction rates

PFR: operate half way between the “optimum” line and the equilibrium line.

- Adiabatic operating line:		xA=−nT0CpnA0ΔHrT−T0

- Non-adiabatic operating line:	dTdxA=yA0QrACp+−ΔHrCp