SEPARATION PROCESSES

- Vapour-Liquid Equilibrium

## Raoult’s Law:

xipi0T=yiP

Where	xi = liquid mole fraction

yi = vapour mole fraction

pi0 = pure component vapour pressure

P = total pressure of the system

## Antoine’s Equation:

log10p0=A−BT+C

Where	p0 = pure component vapour pressure

T = Temperature (C)

A,B,C = constants

In order to calculate the dew point, Td, we combine the two equations above and obtain a rule stating that the sum of the unknown mole fraction is 1. Hence:

x1+x2=1=y1Pp10Td+y2Pp20Td

## Txy phase diagram:

V + L

Liquid phase

Vapour phase

Mole fraction of more volatile component

Temperature (C)

00.20.40.60.81.0

Top line: vapour (dew) point curve

Bottom line: liquid (bubble) point curve

Constant pressure

## McCabe-Thiele Plot:

Equilibrium curve

xA

yA

00.20.40.60.81.0

1.0

2

1

1. Slope = 0  all vapour

2. Slope =   all liquid

Note: 	constant pressure

varying temperature

yeq=α⋅x1+α−1x

## Relative Volatility:

α12=y1/y2x1/x2

Where:	12 	relative vol. of component 1 (m.v.c.) in the vapour phase

α=fT  ideal systems only

## Equilibrium Constant:

ki=yixi

- Multistage Distillation

Vj-1 yj-1

Ljxj

T,

yT

LT

VT

Top operating line (rectifying section):

Overall mass balance:

Vj−1yj−1=Ljxj+TyT

Assuming constant molar overflow:

VTyj−1=LTxj+TyT	(*)

Total balance over top section:

VT=LT+T

Substituting this into (*):

yj−1=LTLT+Txj+TLT+TyT	(**)

Now define the reflux ratio, r:

r=LTT

Hence (**) finally becomes:

yj−1=rr+1xj+1r+1yT		Equation of top operating line

Note:	- 	Slope: rr+1=LTVT

- Intercepts: (yT, yT)

- Maximum value of r =   slope = 1.

- Partial condenser acts as a single theoretical stage.

- Total condenser does NOT act as a single theoretical stage.

## Bottom operating line (stripping section):

Vj yj

Lj+1 xj+1

B, xB

LB

VB

Overall mass balance:

Lj+1xj+1=Vjyj+BxB

Assuming constant molar overflow:

LBxj+1=VByj+BxB	(*)

Total balance over bottom section:

LB=VB+B

Substituting this into (*):

yj=LBVBxj+1−BVBxB

Note:	- 	Slope: LBVB

- Intercepts: (xB, xB)

- Reboiler acts as a single theoretical stage.

## The intersection:

LB

VT

LT

VB

F, zF

The point of intersection is (xq, yq):

VTyq=LTxq+TyT	(a)

VByq=LBxq−BxB	(b)

Subtracting (b) from (a):

yqVT−VB=xqLT−LB+TyT+BxB	(*)

Overall mass balance on the column:

TyT+BxB=FzF

Thus (*) becomes:

yqVT−VB=xqLT−LB+FzF			(**)

Total balance over section:

VT−VB=F+LT−LB

Substituting this into (**):

yq=LT−LBF+LT−LBxq+FF+LT−LBzF	(***)

Now define q:

q=LB−LTF

And finally substitute it into (***):

yq=qqxq−1q−1zF

Note:	- 	Slope: qq−1

- Intercepts: (zF, zF)

### Significance of q:

- Feed at its bubble point (saturated):	q = 1, slope 

- Feed at its dew point (saturated):		q = 0, slope 0

- Mixed vapour-liquid:					0 < q < 1

For feeds between the dew and bubble point, q is the fraction of the feed that is liquid.

Top op. line

Bottom op. line

xA

yA

00.20.40.60.81.0

1.0

3

1

2

q-line

Top composition

Bottom composition

Slope t.o.l.  = r/(r+1)

The final McCabe-Thiele plot:

## Typical distillation calculations:

- Plot equilibrium line.

- Plot xB, yT, xT, and zF.

- Plot q-line.

- Fix reflux ratio (r) and plot top operating line.

- Draw bottom operating line from (xB, xB) to intersection of q-line and top op. line.

- Count number of steps from top to bottom.

- Account for presence of condenser and reboiler.

## Reflux Ratio, r:

Two limiting cases:

- Total reflux  r = 

Number of steps is minimum, but no product formed!

Equilibrium curve

xA

yA

00.20.40.60.81.0

1.0

- Minimum reflux  rmin

Number of steps is  at the pinch point. Operating line crosses equilibrium curve.

Slope of top operating line = rmin/(rmin+1).

Equilibrium curve

xA

yA

00.20.40.60.81.0

1.0

## Steam Injector into the column (replaces reboiler):

Equilibrium curve

xA

yA

xB1.0

1.0

0

xj+1

xj

x2

LB

VB, y0 (=0)

B, xB

yj

yj-1

y1

Stage

j

2

1

0

## Multiple Feeds:

F2

zF,2

T

xT

B

xB

LT

VT

VB

LB

F1

zF,1

LI

VI

xA

yA

xB F2 F1xT

1.0

0

q1

q2

Slope = L1 /VI

Top:		Rectifying section

Middle:	Intermediate section

Bottom:	Stripping section

Mass balance on top section yields:

F1zF1+VIyn−1=TyT+LIxn

	yn−i=LIVIxn+TyTVIyn−FzF1VI

## Column side stream:

F2

zF,2

T

xT

B

xB

LT

VT

VB

LB

FS, xS

Boiling liquid

LI

VI

xA

yA

xB F2 F1xT

1.0

0

q1

q2

Slope = L1 /VI

## Efficiency:

Brought about by poor mixing on trays and thus slow rate of approach to equilibrium.

May vary from tray to tray.

η=nnp

Where:	n = no. of ideal stages

np = no. of actual stages

Murphree Efficiency:

xA

ye

xj1.0

1.0

0

Pseudo

equilibrium line

Actual stage

Ideal stage

yj

yj-1

Emv=yj−yj−1ye−yj−1

ye = composition of vapour in

equil. With liquid xj leaving tray j.

Not necessarily equal to column efficiency.

## Safe operating region:

Factors that affect capacity and can result in sub-optimal/unsafe operation.

- Flooding		- Liquid backs up the downcomer and floods the tray.

- Entrainment	- Incomplete phase separation (e.g. carry-up of liquid droplets)

- Weeping		- Liquid passes down through the holes in tray.

## Distillation column design:

Design considerations:

- Column diameter						 affected by vapour velocity

- No. of trays (depends on efficiency)	 affected by vapour velocity

- Column height

- Tray details

Different approaches:

- Graphical: McCabe-Thiele method

- Empirical: Fenske-Underwood equation

Nmin=logx1−xT/x1−xBlogα

rmin=1α−1yTxF−α1−yt1−xF

Assumptions:	- 	Constant molar overflow

- Ideal mixture  constant relative volatility

- Raoult’s Law applies

- Constant temperature and pressure

## Batch Distillation:

Batch product  small amount, won’t reach steady-state.

yA

1.0

1.0

0

2 stages

yA

1.0

1.0

0

2 stages

Constant product composition							Constant reflux ratio

- Multistage Gas Absorption

  - The presence of a gas stream to remove solute from it.

  - Tray column similar to distillation.

  - Multistage equilibrational diffusional separation process

  - Vapour-liquid equilibrium unsuitable.

  - Inexpensive solvent  high selectivity for the solute.

Solvent liquid

Solvent + solute

Purified gas

Gas + solute

Generally specified:						Want to determine:

- Gas flowrate							- Liquid flowrate

- Composition of inlet gas				- Composition of outlet liquid

- Composition of outlet gas				- Number of stages

Need:

- Operating line (mass balance)

- Equilibrium relationship

## Operating Lines:

LT, xT+1

Lj, xj

GT, yT

Gj-1, yj-1

Balance over element:

Gj−1yj−1=Ljxj+GTyT−LTxT+1

Generally, xT+1 = 0

Transfer of only ONE component

 L and G are NOT constant

(Unless G has v. small conc. of solute)

Removal of trace amount of impurity (or solute)

G and L will be almost constant. Operating line is:

yj−1=LGxj+yT			(if xT+1 = 0)

## Equilibrium relationships:

Determined by solubility of solute (initially in gas phase) in the solvent.

Solution used to recover solute is generally dilute  can use Henry’s Law

yP=HTx

	y=HTxP

Where:	H(T) = Henry’s constant

If isothermal operation  equilibrium is linear

xT+1

xT

xB

yT-1

Equilibrium line: y=(H/P)x

1st stage

2nd stage

yB

Operating line

Operating line ABOVE equilibrium line.

Transfer from gas to liquid

## Changing L/G:

xT+1

xT

xB

yT-1

Equilibrium line: y=(H/P)x

1st st.

2nd st.

Op. line 1

Op. line 2

2nd st.

3rd st.

Hence:	2 stages with high gradient (L/G)

3 stages with low gradient (L/G)

## Minimum value of L/G:

xT+1

xT

x

yT

Equilibrium line

yB

Op. line

Hence:	LG=yB−yTyB/H/P

## Operating Lines (cont.d):

If changes in G and L are significant, use solute free basis:

L’T, XT+1

L’j, Xj

G’T, YT

G’j-1, Yj-1

Y=moles  solutemoles  solvent (gas)

X=moles  solutemoles  solvent

Mass balance:

G'Yj−1=L'Xj+G'YT

	Yj−1=L'G'Xj+YT

Over entire column:

YIN=L'G'XOUT+YOUT

YB=L'G'XB+YT				Slope = L’/G’, intercept (0, YT)

## Analytical methods:

E.g.	Kremser-Souders-Brown equation

Consider:	Dilute solutions

Equilibrium ye = mxe

Can be applied to concentrated solutions if concentrations expressed as mole ratios.

y0−yTy0−mxT+1=AN+1−AAN+1−1						where:	A=LmG

Where:	m = equilibrium constant

N = no. of trays

y0 = conc. in gas phase at inlet

xT+1 = conc. solvent

A = absorption factor

## Thermal Effects:

- Gas solubility decreases with increasing temperature

- Heats of solution often exothermic  temperature increases down the column

This results in a change in equilibrium curve:	 non-linear

 increased number of stages

Isothermal assumption

Real

- Liquid-Liquid Extraction

- To separate liquid mixtures

- To remove a particular component

- Low relative volatility

- Adversely affected by temperature

- Azeotropic mixtures

Mixer-Settler:		Cross-Current

Counter-Current		BETTER

Mass balance  operating line

Equilibrium line  partition coefficient, K

K=conc. of solute in extractconc. of solute in raffinate

X=moles of solutemoles of inert (raffinate  not including solute extract)

x=moles of m.v.c.total moles in stream=moles of solutemoles of raffinate + moles of solute

- Continuous Contact Gas Absorption

- No discrete identifiable stages in which phases equilibrate

- Phases always in contact (continuous contact)

- Flow usually counter-current

- Often involves using a packed tower

- Mass transfer between phases throughout column in the direction of equilibrium

- Required for good operation:	high surface area

low pressure drop

Holding plate

Holding plate

Packing

Liquid distributor

Liquid out

Liquid in

Gas in

Gas out

Two main types of packing:

- STACKED regular, interconnected fixed layers; often arranged as stacks of grids

- RANDOM 	column filed with small discrete packing:

    - Paschrg rings

    - Pall rings

    - Saddles

Major dimension typically 1-10 cm.

## Column Design:

LT, xT

(Solvent)

LB, xB

GT, yT

GB, yB

Packed

Column

Known:	GB, yB

Find:		Solvent flowrate

Diameter of column

Height of column

N = molar flux of solute from gasliquid phase (kmol/m2s)

N

z

Gz+z , yz+z

Gz , yz

Z=H

Z=0

Mass balance on solute:

Gzyz−Gz+δzyz+δz=NδS

Assume G = constant:

- OK is removing trace amounts of solute (y small)

- OK if solute free basis is used

Hence mass balance becomes:

Gyz−yz+δz=NδS		(*)

Now consider r.h.s.:

δS=aAδz

Where a = interfacial area per unit volume (m2/m3)

a=AsV

Now (*) becomes:

Gyz−yz+δz=aANδz

	−Gyz+δz−yzδz=aAN					y|z=0=yB	,	y|z=H=yT

As z  0:

−Gdydz=aAN				(**)

## Gas-Liquid Mass Transfer – the Film Theory:

- Transfer in bulk phases is by convection

- Concentration gradients are negligible except close to the interface

- Transfer in thin films is by diffusion ONLY

- Controlling factor in mass transfer is the rate of diffusion through the thin films

- Concentration at interface are assumed to be at equilibrium

GAS

LIQUID

Thin film

y > yi and x > xi due to direction of transfer

Concentration in gas and liquid at the interface:

yi=yxi		EQUILIBRIUM

Dilute solutions  - use Henry’s Law:

yi=mxi	and		m=HTP

Molar flux through films  controlling factor for diffusion:

N=kGy−yi		(1)		kG = gas side mass transfer coefficient

N=kLx−xi			(2)		kL = liquid side mass transfer coefficient

Interfacial concentrations are generally not available  difficult to measure. Express flux in terms of bulk concentrations:

N=KOGy−yx			(3)		KOG = overall gas phase mass transfer coeff.

	N=KOGy−mx

How is KOG related to kG and kL?

From equation (1):

NkG=y−yi					(4)

And from equation (2):

NkL=xi−x=yim−x		(5)		Assuming Henry’s Law

Multiply (5) by m:

mNkL=yi−mx=yi−y	(6)

Adding equations (4) and (6):

NkG+mNkL=y−y			(7)

Divide l.h.s. of equation (7) by N:

1kG+mkL=y−yN=1KOG

	1KOG=1kG+mkL

A similar procedure can be applied to the liquid side, which yields:

	1KOL=1mkG+1kL

How do we know when to use the liquid or the gas phase transfer equation?

### SPARINGLY SOLUBLE SOLUTE

Mass transfer is liquid side controlled: m is large. Liquid phase analysis is more appropriate:

1KOG≈mkL		and		1KOL≈1kL

### RELATIVELY SOLUBLE SOLUTE

Mass transfer is gas side controlled: m is small. Gas phase analysis is more appropriate:

1KOG≈1kG		and		1KOL≈1mkG

## Column Design (cont.d):

Recall equation (**) obtained on page 15.

−Gdydz=aAN

Now for the gas phase controlled case, assuming A constant::

−Gdydz=aAKOGy−y

To determine height of the tower, rearrange in terms of z:

dz=−GKOGaA⋅dyy−y

Integrate over column height:

dz=−GKOGaA⋅dyy−y

Assume KOG.a.A constant:

H=GKOGaAdyy−y

Where:

HOG=GKOGaA= height required to give one transfer unit of separation (m)

NOG=dyy−y= integrated value of change in composition per unit driving force

NOG represents the difficulty of separation.

## Determination of HOG and NOG:

HOG=GKOGaA

- G and A may be known or part of design requirements.

- A will depend on packing, ay be provided by manufacturers.

- KOG obtained from correlation or as KOG.a from manufacturers.

NOG=dyy−y

Solve by two methods:

G, y

G, yT

L, x

L, xT

Numerical Integration:

yT

yB

1/(y-y*)

Area under curve = NOG

f1

f2

Area under curve:	A=f1+f22Δy+...

Mass balance:	Gy+LxT=Lx+GyT

	y=LGx+yT−LGxT		Operating line	 slope L/G, intercept (xT, yT)

Determination of (y-y*) as a function of y⋅1y−y versus y. can then be used to determine NOG.

yT

xB

yB

y* = f(x)

Op.  line

y-y*

### Analytical method:

Consider special case when equilibrium line is linear	 Henry’s Law applies

 Isothermal operation

NOB=dyy−y		and		y=mx

Then, by algebraic manipulation, it can be shown that:

NOG=yB−yTy−ylm